# Duggaprep

Dugga 1: Kapitel 3-5 förutom 3.7-3.8

Dugga 2: Kapitel 5.6, 6, 7.5, 9, 11.

Dugga 3: 8.1-8.5 samt Beachball Physics

Dugga 4: 10, 12-14 förutom 14.3, 5.7-5.8, 7 förutom 7.5

# Dugga 1

## Kapitel 3

Buffers on the GPU (from back to front):

- Framebuffer object

- Stencil buffer

  - Used for masking

- Double buffer

- Z-buffer

- Frame buffer

Stencil buffer holds integer values, 8bit or 16 unsigned.

To draw a mirrored object using stencil buffer:

- Clear the stencil buffer

- Draw the ground in the stencil

- Draw the mirrored teapot, masked by the stencil

- Draw the ground with some transparency

- Draw the teapot in its standard position

Other stencil buffer uses: Mask out frames, HUDs, dissolve effects, shadow volumes, constructive solid geometry.

glStencilFunc sets the function used to define which areas are masked. For instance glStencilFunc(GL_EQUAL, 1, 0xFFFFFFFF)

glStencilOp decides how the buffer is modified when drawing. It has arguments for stencil fail, stencil pass/depth fail and stencil pass/depth pass. Operations are GL_KEEP, GL_ZERO, GL_REPLACE, GL_INCR, GL_INCR_WRAP, GL_DECR, GL_DECR_WRAP, GL_INVERT.

Continue from 3.4…

## Kapitel 5.6-5.9

New to omdugga, expect to see questions from this.

A large particle system should be processed on the GPU. At a minimum, particles need position and velocity data. These can be passed through the GPU through two textures which should be floating-point buffers.

Use 4 textures and utilize ping-ponging.

Sorting methods…

Geometry shaders

Vertex shader -> Tessellation -> Geometry shader -> (Raster and stuff) -> Fragment shader

Geometry shaders can take as input:

- A line segment (two vertices)

- A line segment with adjacency (four vertices)

- A triangle (three vertices)

- A triangle with adjacency (six vertices)

Geometry shaders can move vertices, or subdivide one vertex into multiple vertices.

Tesselation shaders

Two shaders: tessellation control -> tessellation evaluation, located between vertex and geometry.

# Dugga 3: Fysik

## Kapitel 8.1-8.5

Point masses

Point mass objects are usually represented with: Translation, speed, force accumulator and mass. Sum up the forces, calculate acceleration as a = f/m, update speed from acceleration and position from speed. The simplest way to do this is with Euler integration.

For collision detection, simply check the distance between points. For equal masses: get the new speed after collision by splitting the speed vectors in normal and tangent components, and exchange the normal components.

Animation of rigid bodies

To rotate objects, we need rotation parameters. An object that is rotation symmetric only needs rotation and rotation speed, but other shapes require a model of the moment of inertia.

- X(t) translation (vector)

- R(t) rotation (matrix)

- P(t) linear momentum = integral of r dm (vector)

- L(t) angular momentum^2 = integral of r*x*v dm

- m mass

- J inertia matrix

- Center of mass

To update the state, we need the first derivatives:

- v(t) speed

  - v(t) = P * 1/m

- w*(t)*R(t) angular speed multiplied by rotation

- F(t) linear force

  - Accumulated from external forces

- T(t) torque

  - Cross product of r (vector from center of mass to point of impact) and f

  - To consider friction, split force into a tangent and perpendicular part, change the tangent part

Rotation speed

Rotational angular momentum L = J * ω. ω is rotation speed. In 2d these are scalars, but in 3d L and ω are vectors while J is a matrix.

ω = J^-1 * L

Determining J

J needs to be calculated specifically for each rotation. Start with a “default” J for the shame in its non-rotated position, call it Jbody.

Invert it to obtain Jbody^-1.

Rotate it to match the current orientation of the object in world coordinates:

J^-1 = R*Jbody^-1 * R^T

Friction

The friction force is equal to the opposite of the pushing force, up to a limit. This limit is uk*Fn. There are two different friction coefficients; one for at rest (static) and one in movement (kinetic, lower.) The kinetic friction force is constant.

## Beachball Physics

Movement in discrete time

p(t) = p(t) + v(t)*deltaT

v(t) = v(t) + a(t)*deltaT

Fundamental laws

Newton’s 1st: If no forces act on an object, it will keep the same speed as it has.

Newton’s 2nd: F = ma

Newton’s 3rd: Action and reaction

Conservation of closed systems:

- Σ m (mass) = constant

- Σ E (energy) = constant

- Σ P (momentum) = constant

- Σ L (angular momentum) = constant

Projectiles

Gravity, spin, drag

Collisions

Because of conservation of momentum, v1m1 + v2m2 = a constant (unless external forces are involved.)

Plastic (inelastic) collision

A push – the velocity is recalculated with the mass of both objects involved.

Elastic collision

A bounce – kinetic energy is conserved.

Rotations

annoying

# Dugga 4: AI

Methods for controlling agents:

- Finite-state machine

- Rule-based system

- Planning and problem solving

- Biology inspired AI